# IDI Tech Tip: Using Zebra Fixed RFID Reader Embedded App Compressor

Summary: ID Integration has a growing list of embedded applications to use on the Zebra FX9600 RFID reader.  These apps are text Python or NodeJS text script files which can be edited by the customer.  However, to add them to the FX9600 reader requires that these files be ‘packaged’ and compressed into a ‘Debian’ file for installation through the FX9600 web page interface. This article explains use of the utility that Zebra has created to create this ‘Debian Package File’.

# Software Requirements:

Text Editor, and the file/folder structure show herein.

At https://zebradevs.github.io/rfid-ziotc-docs/user_apps/packaging_and_deployment.html  , there is some more detailed documentation about what the Debian file creates on the reader when installed.

# Build Steps:

Begin by creating a folder structure as shown below:

The DebianBuilder folder is renamed here by IDI, but supplied by Zebra. We have included it in the .zip file that is distributed to customers who have received a custom embedded app from IDI.

The ‘src’ folder is where the python or NodeJS script file is placed.  The ‘pkg’ folder is where configuration files are placed.  In the case of NodeJS apps, this is where the “package.json” file is located.  The ‘out’ folder is where Debian file is created after running the “build.bat” file that IDI has created to make the process more automated.

After adding your program script and configuration file(s) to this structure, run the “build.bat” file and answer the following terminal questions.  When done, your *.deb file will be in the ‘out’ folder.   This is what you will install onto the reader, using the reader’s homepage and the ‘Application’ menu selection .

# NodeJS App Requirements:

Like nearly all Fixed RFID readers, the FX9600 is actually a LinuxOS PC.  It has no display or keyboard.  But it powers on and boots a rather old version of LinuxOS (Debian variant).  Correspondingly, the default installed versions of NodeJS and Python are also quite old and do not support many current software modules that support modern data communication methods.   Lastly, because of the need to securely lock down access to modify the reader OS, there is only a single folder location on the FX9600 where users are allowed at add application files.  The end result is that the reader will not allows users to upgrade or update any system files, including NodeJS and Python.

While this might seem to severely limit the capabilities of the FX9600, there is another option for creating modern NodeJS apps – installing a custom NodeJS modules folder for programs to use instead of the older less capable one that is part of the native OS.  There are numerous complications to creating a custom NodeJS modules folder (node_modules) that will work with that older version of NodeJS on the FX9600.  We won’t go into those details here.  But, we will address how one should download that custom ‘node_modules’ folder to the FX9600.

## File Transfer Tools –

Connecting a WinOS PC to a Linux PC (FX9600) can be done with a variety of tools:  PuTTY, WinSCP, or any FTP program.   PuTTY is purely terminal command line driven, whereas WinSCP, Filezilla, or CoreFTP offer drop and drag functionality.

Using WinSCP, one simply logs into the reader IP using username ‘rfidadm’ with no password.  Then select the custom ‘node_modules’ folder and drag it to the FX9600 ‘/app/’ folder.   The folder is fairly large and does require some time for the download.   This size is the reason why it can’t be included in the Debian compressor utility process.

Using PuTTY, one needs to open the PuTTY folder, usually at C:\Program Files\PuTTY\.   Once there, open a terminal window by right clicking in the open space within that file manager window.  Then issue the following command, putting your location for the node_modules folder in the command structure shown in this video https://youtu.be/nexXUGnpWwc .